# MODULE 1: INTRODUCTION

### DATA COMMUNICATIONS

- Data Communication is defined as exchange of data between 2 devices over a transmission-medium.

- A communication-system is made up of

→ hardware (physical equipment) and

→ software (programs)

- For data-communication, the communicating-devices must be part of a communication-system.

- Four attributes of a communication-system:

### Delivery

  - The system must deliver data to the correct destination.

### Accuracy

  - The system must deliver the data accurately.

  - Normally, the corrupted-data are unusable.

### Timeliness

  - The system must deliver audio/video data in a timely manner.

  - This kind of delivery is called real-time transmission.

  - Data delivered late are useless.

### Jitter

  - Jitter refers to the variation in the packet arrival-time.

  - In other words, jitter is the uneven delay in the delivery of audio/video packets.

### Components of Communication System

- Five components of a communication-system (Figure 1.1):

      - Message

      - Sender

      - Receiver

      - Transmission-Medium

      - Protocol

### Message

  - Message is the information (or data) to be communicated.

  - Message may consist of

→ number/text

→ picture or

→ audio/video

### Sender

  - Sender is the device that sends the data-message.

  - Sender can be

→ computer and

→ mobile phone

### Receiver

  - Receiver is the device that receives the message.

  - Receiver can be

→ computer and

→ mobile phone

### Transmission Medium

  - Transmission-medium is physical-path by which a message travels from sender to receiver.

  - Transmission-medium can be wired or wireless.

  - Examples of wired medium:

→ twisted-pair wire (used in landline telephone)

→ coaxial cable (used in cable TV network)

→ fiber-optic cable

  - Examples of wireless medium:

→ radio waves

→ microwaves

→ infrared waves (ex: operating TV using remote control)

### Protocol

  - A protocol is a set of rules that govern data-communications.

  - In other words, a protocol represents an agreement between the communicating-devices.

  - Without a protocol, 2 devices may be connected but not communicating.

### Data Representation

- Five different forms of information:

### Text

  - Text is represented as a bit-pattern. (Bit-pattern  sequence of bits: 0s or 1s).

  - Different sets of bit-patterns are used to represent symbols (or characters).

  - Each set is called a code.

  - The process of representing symbols is called encoding.

  - Popular encoding system: ASCII, Unicode.

### Number

  - Number is also represented as a bit-pattern.

  - ASCII is not used to represent number. Instead, number is directly converted to binary-form.

### Image

  - Image is also represented as a bit-pattern.

  - An image is divided into a matrix of pixels (picture-elements).

  - A pixel is the smallest element of an image. (Pixel  Small dot)

  - The size of an image depends upon number of pixels (also called resolution).

For example: An image can be divided into 1000 pixels or 10,000 pixels.

  - Two types of images:

### Black & White Image

¤ If an image is black & white, each pixel can be represented by a value either 0 or 1.

¤ For example: Chessboard

### Color Image

¤ There are many methods to represent color images.

¤ RGB is one of the methods to represent color images.

¤ RGB is called so called ‘.’ each color is combination of 3 colors: red, green & blue.

### Audio

  - Audio is a representation of sound.

  - By nature, audio is different from text, numbers, or images. Audio is continuous, not discrete.

### Video

  - Video is a representation of movie.

  - Video can either

→ be produced as a continuous entity (e.g., by a TV camera), or

→ be a combination of images arranged to convey the idea of motion.

### Direction of Data Flow

- Three ways of data-flow between 2 devices (Figure 1.2):

      - Simplex

      - Half-duplex

      - Full-duplex

### Simplex

  - The communication is unidirectional

(For ex: The simplex mode is like a one-way street).

  - On a link, out of 2 devices:

  - Only one device can transmit.

  - Another device can only receive.

  - For example (Figure 1.2a):

The monitor can only accept output.

  - Entire-capacity of channel is used to send the data in one direction.

### Half-Duplex

  - Both the stations can transmit as well as receive but not at the same time.

(For ex: The half-duplex mode is like a one-lane road with 2 directional traffic).

  - When one station is sending, the other can only receive and vice-versa.

  - For example (Figure 1.2b): Walkie-talkies

  - Entire-capacity of a channel is used by one of the 2 stations that are transmitting the data.

### Full-Duplex

  - Both stations can transmit and receive at the same time.

(For ex: The full-duplex is like a 2-way street with traffic flowing in both directions at the same time).

  - For example (Figure 1.2c):

Mobile phones (When 2 people are communicating by a telephone line, both can listen and talk at the same time)

  - Entire-capacity of a channel is shared by both the stations that are transmitting the data.

### NETWORKS

- A network is defined as a set of devices interconnected by communication-links.

- This interconnection among computers facilitates information sharing among them.

- Computers may connect to each other by either wired or wireless media.

- Often, devices are referred to as nodes.

- A node can be any device capable of sending/receiving data in the network.

- For example: Computer & Printer

- The best-known computer network is the Internet.

### Network Criteria

- A network must meet following 3 criteria’s:

### Performance

  - Performance can be measured using i) Transit-time or ii) Response-time.

        - Transit Time is defined as time taken to travel a message from one device to another.

        - Response Time is defined as the time elapsed between enquiry and response.

  - The network-performance depends on following factors:

          - Number of users

          - Type of transmission-medium

          - Efficiency of software

  - Often, performance is evaluated by 2 networking-metrics: i) throughput and ii) delay.

  - Good performance can be obtained by achieving higher throughput and smaller delay times

### Reliability

  - Reliability is measured by

→ frequency of network-failure

→ time taken to recover from a network-failure

→ network's robustness in a disaster

  - More the failures are, less is the network's reliability.

### Security

  - Security refers to the protection of data from the unauthorized access or damage.

  - It also involves implementing policies for recovery from data-losses.

### Physical Structures 1.2.2.1 Type of Connection

- Two types of connections (Figure 1.3):

### Point-to-Point

  - Only two devices are connected by a dedicated-link (Figure 1.3a).

  - Entire-capacity of the link is reserved for transmission between those two devices.

  - For example: Point-to-Point connection b/w remote-control & TV for changing the channels.

### Multipoint (Multi-drop)

  - Three or more devices share a single link.

  - The capacity of the channel is shared, either spatially or temporally (Figure 1.3b).

        - If link is used simultaneously by many devices, then it is spatially shared connection.

        - If user takes turns while using the link, then it is time shared (temporal) connection.

(spatiallyspace or temporallytime)

### Physical Topology

- The physical-topology defines how devices are connected to make a network.

- Four basic topologies are:

        - Mesh

        - Star

        - Bus and

        - Ring

### Bus Topology

- All the devices are connected to the single cable called bus (Figure 1.4).

- Every device communicates with the other device through this bus.

- A data from the source is broadcasted to all devices connected to the bus.

- Only the intended-receiver, whose physical-address matches, accepts the data.

- Devices are connected to the bus by drop-lines and taps.

- A drop-line is a connection running between the device and the bus.

- A tap is a connector that links to the bus or

- Advantages:

          - Easy installation.

          - Cable required is the least compared to mesh/star topologies.

          - Redundancy is eliminated.

          - Costs less (Compared to mesh/star topologies).

          - Mostly used in small networks. Good for LAN.

- Disadvantages:

- Difficult to detect and troubleshoot fault.

- Signal reflection at the taps can cause degradation in quality.

- A fault/break in the cable stops all transmission.

- There is a limit on

  - Cable length

  - Number of nodes that can be connected.

- Security is very low because all the devices receive the data sent from the source.

### Star Topology

- All the devices are connected to a central controller called a hub (Figure 1.5).

- There exists a dedicated point-to-point link between a device & a hub.

- The devices are not directly linked to one another. Thus, there is no direct traffic between devices.

- The hub acts as a junction:

If device-1 wants to send data to device-2,

the device-1 sends the data to the hub,

then the hub relays the data to the device-2.

- Advantages:

          - Less expensive: Each device needs only one link & one I/O port to connect it to any devices.

          - Easy installation & reconfiguration: Nodes can be added/removed w/o affecting the network.

          - Robustness: If one link fails, it does not affect the entire system.

          - Easy to detect and troubleshoot fault.

          - Centralized management: The hub manages and controls the whole network.

- Disadvantages:

- Single point of failure: If the hub goes down, the whole network is dead.

- Cable length required is the more compared to bus/ring topologies.

- Number of nodes in network depends on capacity of hub.

### Ring Topology

- Each device is connected to the next, forming a ring (Figure 1.6).

- There are only two neighbors for each device.

- Data travels around the network in one direction till the destination is reached.

- Sending and receiving of data takes place by the help of token.

- Each device has a repeater.

- A repeater

→ receives a signal on transmission-medium &

→ regenerates & passes the signal to next device.

- Advantages:

          - Easy installation and reconfiguration.

To add/delete a device, requires changing only 2 connections.

3) Fault isolation is simplified.

If one device does not receive a signal within a specified period, it can issue an alarm.

The alarm alerts the network-operator to the problem and its location.

3) Congestion reduced: Because all the traffic flows in only one direction.

- Disadvantages:

- Unidirectional traffic.

- A fault in the ring/device stops all transmission.

The above 2 drawbacks can be overcome by using dual ring.

- There is a limit on

  - Cable length &

  - Number of nodes that can be connected.

- Slower: Each data must pass through all the devices between source and destination.

### Mesh Topology

- All the devices are connected to each other (Figure 1.7).

- There exists a dedicated point-to-point link between all devices.

- There are n(n-1) physical channels to link n devices.

- Every device not only sends its own data but also relays data from other nodes.

- For ‘n’ nodes,

→ there are n(n-1) physical-links

→ there are n(n-1)/2 duplex-mode links

- Every device must have (n–1) I/O ports to be connected to the other (n-1) devices.

- Advantages:

          - Congestion reduced: Each connection can carry its own data load.

          - Robustness: If one link fails, it does not affect the entire system.

          - Security: When a data travels on a dedicated-line, only intended-receiver can see the data.

          - Easy fault identification & fault isolation: Traffic can be re-routed to avoid problematic links.

- Disadvantages:

- Difficult installation and reconfiguration.

- Bulk of wiring occupies more space than available space.

- Very expensive: as there are many redundant connections.

- Not mostly used in computer networks. It is commonly used in wireless networks.

- High redundancy of the network-connections.

### Network Types

- Two popular types of networks:

- LAN (Local Area Network) &

- WAN (Wide Area Network)

### LAN

- LAN is used to connect computers in a single office, building or campus (Figure 1.8).

- LAN is usually privately owned network.

- A LAN can be simple or complex.

      - Simple: LAN may contain 2 PCs and a printer.

      - Complex: LAN can extend throughout a company.

- Each host in a LAN has an address that uniquely defines the host in the LAN.

- A packet sent by a host to another host carries both source host’s and destination host’s addresses.

- LANs use a smart connecting switch.

- The switch is able to

→ recognize the destination address of the packet &

→ guide the packet to its destination.

- The switch

→ reduces the traffic in the LAN &

→ allows more than one pair to communicate with each other at the same time.

- Advantages:

### Resource Sharing

  - Computer resources like printers and hard disks can be shared by all devices on the network.

### Expansion

  - Nowadays, LANs are connected to WANs to create communication at a wider level.

### WAN

- WAN is used to connect computers anywhere in the world.

- WAN can cover larger geographical area. It can cover cities, countries and even continents.

- WAN interconnects connecting devices such as switches, routers, or modems.

- Normally, WAN is

→ created & run by communication companies (Ex: BSNL, Airtel)

→ leased by an organization that uses it.

- A WAN can be of 2 types:

### Point-to-point WAN

  - A  point-to-point WAN is  a  network that connects 2  communicating devices through  a transmission media (Figure 1.9).

### Switched WAN

  - A switched WAN is a network with more than two ends.

  - The switched WAN can be the backbones that connect the Internet.

  - A switched WAN is a combination of several point-to-point WANs that are connected  by switches (Figure 1.10).

### Internetwork

- A network of networks is called an internet. (Internet  inter-network) (Figure 1.12).

- For example (Figure 1.11):

  - Assume that an organization has two offices,

        - First office is on the east coast &

        - Second office is on the west coast.

  - Each office has a LAN that allows all employees in the office to communicate with each other.

  - To allow communication between employees at different offices, the management leases a point-to-point dedicated WAN from a ISP and connects the two LANs.

(ISP  Internet service provider such as a telephone company ex: BSNL).

  - When a host in the west coast office sends a message to another host in the same office, the router blocks the message, but the switch directs the message to the destination.

  - On the other hand, when a host on the west coast sends a message to a host on the east coast, router R1 routes the packet to router R2, and the packet reaches the destination.

### LAN vs. WAN

| Parameters | LAN | WAN |
| --- | --- | --- |
| Expands to | Local Area Network | Wide Area Network |
| Meaning | LAN is used to connect computers in a single office, building or campus | WAN is used to connect computers in a large geographical area such as countries |
| Ownership of network | Private | Private or public |
| Range | Small: up to 10 km | Large: Beyond 100 km |
| Speed | High: Typically 10, 100 and 1000 Mbps | Low: Typically 1.5 Mbps |
| Propagation Delay | Short | Long |
| Cost | Low | High |
| Congestion | Less | More |
| Design & maintenance | Easy | Difficult |
| Fault Tolerance | More Tolerant | Less Tolerant |
| Media used | Twisted pair | Optical fiber or radio waves |
| Used for | College, Hospital | Internet |
| Interconnects | LAN interconnects hosts | WAN interconnects connecting devices such as switches, routers, or modems |

### Switching

- An internet is a switched network in which a switch connects at least two links together.

- A switch needs to forward data from a network to another network when required.

- Two types of switched networks are 1) circuit-switched and 2) packet-switched networks.

### Circuit-Switched Network

- A dedicated connection, called a circuit, is always available between the two end systems.

- The switch can only make it active or inactive.

¤ As shown in Figure 1.13, the 4 telephones at each side are connected to a switch.

¤ The switch connects a telephone at one side to a telephone at the other side.

¤ A high-capacity line can handle 4 voice communications at the same time.

¤ The capacity of high line can be shared between all pairs of telephones.

¤ The switch is used for only forwarding.

- Advantage:

A circuit-switched network is efficient only when it is working at its full capacity.

- Disadvantage:

Most of the time, the network is inefficient because it is working at partial capacity.

### Packet-Switched Network

- In a computer network, the communication between the 2 ends is done in blocks of data called packets.

- The switch is used for both storing and forwarding because a packet is an independent entity

that can be stored and sent later.

¤ As shown in Figure 1.14, the 4 computers at each side are connected to a router.

¤ A router has a queue that can store and forward the packet.

¤ The high-capacity line has twice the capacity of the low-capacity line.

¤ If only 2 computers (one at each site) need to communicate with each other, there is no waiting for the packets.

¤ However, if packets arrive at one router when high-capacity line is at its full capacity, the packets should be stored and forwarded.

- Advantages:

A packet-switched network is more efficient than a circuit switched network.

- Disadvantage:

The packets may encounter some delays.

### The Internet Today

- A network of networks is called an internet. (Internet  inter-network)

- Internet is made up of (Figure 1.15)

- Backbones

- Provider networks &

- Customer networks

### Backbones

  - Backbones are large networks owned by communication companies such as BSNL and Airtel.

  - The backbone networks are connected through switching systems, called peering points.

### Provider Networks

  - Provider networks use the services of the backbones for a fee.

  - Provider networks are connected to backbones and sometimes to other provider networks.

### Customer Networks

  - Customer networks actually use the services provided by the Internet.

  - Customer networks pay fees to provider networks for receiving services.

- Backbones and provider networks are also called Internet Service Providers (ISPs).

- The backbones are often referred to as international ISPs.

The provider networks are often referred to as national or regional ISPs.

### Accessing the Internet

- The Internet today is an internetwork that allows any user to become part of it.

- However, the user needs to be physically connected to an ISP.

- The physical connection is normally done through a point-to-point WAN.

### Using Telephone Networks

  - Most residences have telephone service, which means they are connected to a  telephone network.

  - Most telephone networks have already connected themselves to the Internet.

  - Thus, residences can connect to the Internet using a point-to-point WAN.

  - This can be done in two ways:

### Dial-up service

¤ A modem can be added to the telephone line.

¤ A modem converts data to voice.

¤ The software installed on the computer

→ dials the ISP &

→ imitates making a telephone connection.

¤ Disadvantages:

    - The dial-up service is very slow.

    - When  line  is  used  for  Internet  connection, it  cannot  be  used  for  voice connection.

    - It is only useful for small residences.

### DSL Service

¤  DSL  service  also  allows  the  line  to  be  used  simultaneously  for  voice  &  data communication.

¤ Some telephone companies have upgraded their telephone lines to provide higher

speed Internet services to residences.

### Using Cable Networks

  - A residence can be connected to the Internet by using cable service.

  - Cable service provides a higher speed connection.

  - The speed varies depending on the number of neighbors that use the same cable.

### Using Wireless Networks

  - A residence can use a combination of wireless and wired connections to access the Internet.

  - A residence can be connected to the Internet through a wireless WAN.

### Direct Connection to the Internet

  - A large organization can itself become a local ISP and be connected to the Internet.

  - The organization

→ leases a high-speed WAN from a carrier provider and

→ connects itself to a regional ISP.

# MODULE 1(CONT.): NETWORK MODELS

### PROTOCOL LAYERING

- A protocol defines the rules that both the sender and receiver and all intermediate devices need to follow to be able to communicate effectively.

- When communication is simple, we may need only one simple protocol.

When communication is complex, we need to divide the task b/w different layers. We need a protocol at each layer, or protocol layering.

### Scenarios First Scenario

- In the first scenario, communication is so simple that it can occur in only one layer (Figure 2.1).

- Assume Maria and Ann are neighbors with a lot of common ideas.

- Communication between Maria and Ann takes place in one layer, face to face, in the same language

### Second Scenario

- Maria and Ann communicate using regular mail through the post office (Figure 2.2).

- However, they do not want their ideas to be revealed by other people if the letters are intercepted.

- They agree on an encryption/decryption technique.

- The sender of the letter encrypts it to make it unreadable by an intruder; the receiver of the letter

decrypts it to get the original letter.

### Protocol Layering

- Protocol layering enables us to divide a complex task into several smaller and simpler tasks.

- Modularity means independent layers.

- A layer (module) can be defined as a black box with inputs and outputs, without concern about how inputs are changed to outputs.

- If two machines provide the same outputs when given the same inputs, they can replace each other.

- Advantages:

        - It allows us to separate the services from the implementation.

        - There are intermediate systems that need only some layers, but not all layers.

- Disadvantage:

1) Having a single layer makes the job easier. There is no need for each layer to provide a service to the upper layer and give service to the lower layer.

### Principles of Protocol Layering

### First Principle

- If we want bidirectional communication, we need to make each layer able to perform 2  opposite tasks, one in each direction.

- For example, the third layer task is to listen (in one direction) and talk (in the other direction).

### Second Principle

- The two objects under each layer at both sites should be identical.

- For example, the object under layer 3 at both sites should be a plaintext letter.

### Logical Connections

- We have layer-to-layer communication (Figure 2.3).

- There is a logical connection at each layer through which 2 end systems can send the object created from that layer.

### TCP/IP PROTOCOL SUITE

- TCP/IP is a protocol-suite used in the Internet today.

- Protocol-suite refers a set of protocols organized in different layers.

- It is a hierarchical protocol made up of interactive modules, each of which provides a  specific functionality.

- The term hierarchical means that each upper level protocol is supported by the services provided by

one or more lower level protocols.

### Layered Architecture

- Let us assume that computer A communicates with computer B (Figure 2.4).

- As the Figure 2.5 shows, we have five communicating devices:

1) Source host(computer A)	2) Link-layer switch in link 1

3) Router	4) Link-layer switch in link 2

5) Destination host (computer B).

- Each device is involved with a set of layers depending on the role of the device in the internet.

- The two hosts are involved in all five layers.

- The source host

→ creates a message in the application layer and

→ sends the message down the layers so that it is physically sent to the destination host.

- The destination host

→ receives the message at the physical layer and

→ then deliver the message through the other layers to the application layer.

- The router is involved in only three layers; there is no transport or application layer.

- A router is involved in n combinations of link and physical layers.

where n = number of links the router is connected to.

- The reason is that each link may use its own data-link or physical protocol.

- A link-layer switch is involved only in two layers: i) data-link and ii) physical.

### Layers in the TCP/IP Protocol Suite

- As shown in the figure 2.6, the duty of the application, transport, and network layers is end-to-end.

- However, the duty of the data-link and physical layers is hop-to-hop. A hop is a host or router.

- The domain of duty of the top three layers is the internet.

The domain of duty of the two lower layers is the link.

- In top 3 layers, the data unit should not be changed by any router or link-layer switch.

In bottom 2 layers, the data unit is changed only by the routers, not by the link-layer switches.

- Identical objects exist between two hops. Because router may fragment the packet at the network layer and send more packets than received (Figure 2.7).

- The link between two hops does not change the object.

### Description of Each Layer Physical Layer

- The physical layer is responsible for movements of individual bits from one node to another node.

- Transmission media is another hidden layer under the physical layer.

- Two devices are connected by a transmission medium (cable or air).

- The transmission medium does not carry bits; it carries electrical or optical signals.

- The physical layer

→ receives bits from the data-link layer &

→ sends through the transmission media.

### Data Link Layer

- Data-link-layer (DLL) is responsible for moving frames from one node to another node over a link.

- The link can be wired LAN/WAN or wireless LAN/WAN.

- The data-link layer

→ gets the datagram from network layer

→ encapsulates the datagram in a packet called a frame.

→ sends the frame to physical layer.

- TCP/IP model does not define any specific protocol.

- DLL supports all the standard and proprietary protocols.

- Each protocol may provide a different service.

- Some protocols provide complete error detection and correction; some protocols provide only error correction.

### Network Layer

- The network layer is responsible for source-to-destination transmission of data.

- The network layer is also responsible for routing the packet.

- The routers choose the best route for each packet.

- Why we need the separate network layer?

- The separation of different tasks between different layers.

- The routers do not need the application and transport layers.

- TCP/IP model defines 5 protocols:

1) IP (Internetworking Protocol)	2) ARP (Address Resolution Protocol)

3) ICMP (Internet Control Message Protocol)	4) IGMP (Internet Group Message Protocol)

### IP

  - IP is the main protocol of the network layer.

  - IP defines the format and the structure of addresses.

  - IP is also responsible for routing a packet from its source to its destination.

  - It is a connection-less & unreliable protocol.

  - Connection-less means there is no connection setup b/w the sender and the receiver.

  - Unreliable protocol means

→ IP does not make any guarantee about delivery of the data.

→ Packets may get dropped during transmission.

  - It provides a best-effort delivery service.

  - Best effort means IP does its best to get the packet to its destination, but with no guarantees.

  - IP does not provide following services

→ flow control

→ error control

→ congestion control services.

  - If an application requires above services, the application should rely only on the transport- layer protocol.

### ARP

  - ARP is used to find the physical-address of the node when its Internet-address is known.

  - Physical address is the 48-bit address that is imprinted on the NIC or LAN card.

  - Internet address (IP address) is used to uniquely & universally identify a device in  the internet.

### ICMP

  - ICMP is used to inform the sender about datagram-problems that occur during transit.

### IGMP

  - IGMP is used to send the same message to a group of recipients.

### Transport Layer

- TL protocols are responsible for delivery of a message from a process to another process.

- The transport layer

→ gets the message from the application layer

→ encapsulates the message in a packet called a segment and

→ sends the segment to network layer.

- TCP/IP model defines 3 protocols:1) TCP (Transmission Control Protocol)

- UDP (User Datagram Protocol) &

- SCTP (Stream Control Transmission Protocol)

### TCP

  - TCP is a reliable connection-oriented protocol.

  - A connection is established b/w the sender and receiver before the data can be transmitted.

  - TCP provides

→ flow control

→ error control and

→ congestion control

### UDP

  - UDP is the simplest of the 3 transport protocols.

  - It is an unreliable, connectionless protocol.

  - It does not provide flow, error, or congestion control.

  - Each datagram is transported separately & independently.

  - It is suitable for application program that

→ needs to send short messages &

→ cannot afford the retransmission.

### SCTP

  - SCTP provides support for newer applications such as voice over the Internet.

  - It combines the best features of UDP and TCP.

### Application Layer

- The two application layers exchange messages between each other.

- Communication at the application layer is between two processes (two programs running at  this layer).

- To communicate, a process sends a request to the other process and receives a response.

- Process-to-process communication is the duty of the application layer.

- TCP/IP model defines following protocols:

- SMTP is used to transport email between a source and destination.

- TELNET is used for accessing a site remotely.

- FTP is used for transferring files from one host to another.

- DNS is used to find the IP address of a computer.

- SNMP is used to manage the Internet at global and local levels.

- HTTP is used for accessing the World Wide Web (WWW).

(FTP  File Transfer Protocol	SMTP  Simple Mail Transfer Protocol) (DNS  Domain Name System	HTTP  Hyper Text Transfer Protocol) (SNMP  Simple Network Management Protocol	TELNET  Terminal Network)

### Encapsulation and Decapsulation

### Encapsulation at the Source Host

- At the source, we have only encapsulation (Figure 2.8).

  - At the application layer, the data to be exchanged is referred to as a message.

  - A message normally does not contain any header or trailer.

  - The message is passed to the transport layer.

  - The transport layer takes the message as the payload.

  - TL adds its own header to the payload.

  - The header contains

→ identifiers of the source and destination application programs

→ information needed for flow, error control, or congestion control.

  - The transport-layer packet is called the segment (in TCP) and the user datagram (in UDP).

  - The segment is passed to the network layer.

  - The network layer takes the transport-layer packet as payload.

  - NL adds its own header to the payload.

  - The header contains

→ addresses of the source and destination hosts

→ some information used for error checking of the header &

→ fragmentation information.

  - The network-layer packet is called a datagram.

  - The datagram is passed to the data-link layer.

  - The data-link layer takes the network-layer packet as payload.

  - DLL adds its own header to the payload.

  - The header contains the physical addresses of the host or the next hop (the router).

  - The link-layer packet is called a frame.

  - The frame is passed to the physical layer for transmission

### Decapsulation and Encapsulation at the Router

- At the router, we have both encapsulation & encapsulation and because the router is connected to two or more links.

  - Data-link layer

→ receives frame from physical layer

→ decapsulates the datagram from the frame and

→ passes the datagram to the network layer.

  - The network layer

→ inspects the source and destination addresses in the datagram header and

→ consults forwarding table to find next hop to which the datagram is to be delivered.

  - The datagram is then passed to the data-link layer of the next link.

  - The data-link layer of the next link

→ encapsulates the datagram in a frame and

→ passes the frame to the physical layer for transmission.

### Decapsulation at the Destination Host

- At the destination host, each layer

→ decapsulates the packet received from lower layer

→ removes the payload and

→ delivers the payload to the next-higher layer

### Addressing

- We have logical communication between pairs of layers.

- Any communication that involves 2 parties needs 2 addresses: source address and  destination address.

- We need 4 pairs of addresses (Figure 2.9):

- At the application layer, we normally use names to define

→ site that provides services, such as vtunotesbysri.com, or

→ e-mail address, such as vtunotesbysree@gmail.com.

- At the transport layer, addresses are called port numbers.

  - Port numbers define the application-layer programs at the source and destination.

  - Port numbers are local addresses that distinguish between several programs running at the same time.

- At the network-layer, addresses are called IP addresses.

  - IP address uniquely defines the connection of a device to the Internet.

  - The IP addresses are global, with the whole Internet as the scope.

- At the data link-layer, addresses are called MAC addresses

  - The MAC addresses defines a specific host or router in a network (LAN or WAN).

  - The MAC addresses are locally defined addresses.

### Multiplexing and Demultiplexing

- Multiplexing means a protocol at a layer can encapsulate a packet from several next-higher layer protocols (one at a time) (Figure 2.10).

- Demultiplexing means a protocol can decapsulate and deliver a packet to several next-higher layer

protocols (one at a time).

- At transport layer, either UDP or TCP can accept a message from several application-layer protocols.

- At network layer, IP can accept

→ a segment from TCP or a user datagram from UDP.

→ a packet from ICMP or IGMP.

- At data-link layer, a frame may carry the payload coming from IP or ARP.

### OSI MODEL

- OSI model was developed by ISO.

- ISO is the organization, OSI is the model.

- Purpose: OSI was developed to allow systems with diff. platforms to communicate with each other.

- Platform means hardware, software or operating system.

- OSI is a network-model that defines the protocols for network communications.

- OSI has 7 layers as follows (Figure 2.11):

- Application Layer

- Presentation Layer

- Session Layer

- Transport Layer

- Network Layer

- Data Link Layer

- Physical Layer

- Each layer has specific duties to perform and has to co-operate with the layers above & below it.

### OSI vs. TCP/IP

- The four bottommost layers in the OSI model & the TCP/IP model are same (Figure 2.12).

However, the Application-layer of TCP/IP model corresponds to the Session, Presentation

& Application Layer of OSI model.

Two reasons for this are:

  - TCP/IP has more than one transport-layer protocol.

  - Many applications can be developed at Application layer

- The OSI model specifies which functions belong to each of its layers.

In TCP/IP model, the layers contain relatively independent protocols that can be mixed and matched depending on the needs of the system.

### Lack of OSI Model’s Success

- OSI was completed when TCP/IP was fully in place and a lot of time and money had been spent on the suite; changing it would cost a lot.

- Some layers in the OSI model were never fully defined.

- When OSI was implemented by an organization in a different application, it did not show a  high

enough level of performance